Packet Hub

159, Jalan Templer

46050 Petaling Jaya

Selangor Darul Ehsan

Malaysia.

Tel:  +60-3-74508888

Fax: +60-3-74508899

Quick User Guide

For DX WIMAX Indoor Modem

Prepared by:

Green Packet Berhad

Date:

November 24, 2010

Document Information

| Title: | DX WiMAX Indoor Modem Quick User Guide | Prepared by: | Khor Hock Lin |
| --- | --- | --- | --- |
| Our Reference #: |  | Approved by: |  |
| Document Version: | Rev 1.0 | Document Date: | Nov. 24, 2010 |

Document Distribution/To

| Company | Purpose** | Attention | Tel/Fax/Email |
| --- | --- | --- | --- |

Updated Document

| Rev. | Release Date | Updated By | Approved by | Description of Change |
| --- | --- | --- | --- | --- |

TABLE OF CONTENTS

1.	GENERAL	4

2.	GUI INTERFACE CONNECTION	5

3.	CONFIGURATION DX INDOOR MODEM	7

3.1.	Status of WiMAX	7

3.2.	WiMAX Frequency Configuration	8

3.3.	WiMAX Authentication Configuration	9

3.4.	WIFI Configuration	10

3.5.	Firmware Upgrade	12

3.6.	Recovery	13

3.7.	Logout and Reboot	14

3.7.1.	Logout	14

3.7.2.	Reboot	14

# GENERAL

This quick user guide is intended as a reference source of configuring the DX indoor modem.

Hardware Requirements

The hardware requirements of DX are shown in the table below:-

| No. | Descriptions | Specifications | Qty |
| --- | --- | --- | --- |
| A | Hardware Requirement |  |  |
| 1 | DX WiMAX Indoor Modem | c/w Power Adapter and RJ45 Network cable | 1 set |

# ACCESS MODEM GUI

Follow the steps below to login to DX GUI through your pc or laptop. Prior to configuration, please ensure that the above mentioned requirements are met.

Web Browser to Login into DX GUI

- Use the network cable provided to connect DX WiMAX Indoor Modem to the LAN port of your laptop or PC.

- Power ON and allow the laptop or PC to detect the network connection.

- Double-click on Internet Explorer, go to http://10.1.1.254 and login with the following default credential:

- Username: admin

- Password: admin

If Internet Explorer cannot display the webpage

Please follow the following instruction:-

- Run command prompt (press Window button  + r button) and key in “cmd”

- Key in “ipconfig” to check the status of  local LAN network information

Default Gateway IP address

- Double-click on Internet Explorer, go to http://x.x.x.x and login with the following default credential:

- Username: admin

- Password: admin

# DX INDOOR MODEM CONFIGURATION

## Status

### WiMAX Status

WiMAX Status

Click on “Status” on the top and “WiMAX Status” on the left, this tab gives you a summary of WiMAX status, such as the Frequency, BSID, RSSI, CINR and etc.

### Network Status

Network Status

Click on “Status” on the top and “Network Status” on the left, this tab gives you a summary of Network status, such as IP address, Gateway, Mac address and etc.

## WiMAX Frequency Configuration

### Scanner

To Edit Frequency & Bandwidth

WiMAX Scanner

- Click on “WiMAX” on the top and “Scanner” on the left, you can start or stop the WiMAX connectivity.

- This Scanner tab, a list of BS which the CPE detected will be displayed in Channel Table.

- Press Edit button, can set insert and delete the Channel Frequency (KHz) and Bandwidth (MHz).

Insert and Delete Frequency

- Finally, click Apply button to save the configuration.

Note: set Frequency and Bandwidth is base on local WiMAX license.

## WiMAX Authentication Configuration

WiMAX Authentication

The following is the instruction of configuration:-

- This Authentication tab, configure the authentication settings for the WiMAX network.

- Choose the EAP type of Phase 1 and Phase 2 which your AAA is configured from EAP Method.

- Key in User Name and Password configured in AAA.

- Key in Identity ID

- Click Browse button at the CA Certificate and browse to the path where the CA Certificate is located.

- Finally, click the Apply button to active the configured profile.

## WIFI Configuration

WIFI Network

The following is the instruction of configuration:-

- This WIFI tab, to configure the wireless settings for the end user network. End user can access the internet thru wireless connectivity.

- First, choose the wireless technologies mode as 802.11b/g/n mixed.

- Select Auto channel scanning.

- Make the transmit (Tx) power and burst is 100% and Enable.

- Click Next button to configure wireless network profile.

- Select User Defined as default.

- Create a wireless network name, known as SSID.

- Create wireless encryption (WEP) or (WiFi Protected Access (WPA), which help protect your wireless network.

- Finally, click Apply button save the wireless network configuration.

- System will auto re-connecting the setup after 10 seconds.

## Firmware Upgrade

Management Upgrade

- This Upgrade tab, click browse button at the Firmware Image File and browse to the path where the Firmware is located.

Note: sample of file type: - xx_xx_ver x.x.x.x.tar.gz

## Recovery

Management Recovery

- This Recovery tab is reset all the configuration and setting to factory default.

## Logout and Reboot

Logout

Logout

- This Logout tab is log off the user process at its end.

Reboot

Reboot

- Click Reboot button to restart CPE.

- System will reboot CPE and redirecting to login after 60 seconds.

End Document